LED 36 beam dyeing light manual

Voltage: AC90-240V 50/60HZ

Total power supply; 180W

Channel: 9 and 16 channels optional

Light source; Jingxin lamp beads 36 X3W, 8 red; 10 green; 10 blue; 8 white;

X axis; 540 degrees,

Y axis 270 degrees

DMX SET

001-512

Channel SET

9

512 DMX SET

9 channel

16

16cahnnel

Show Mode

Dmx Mode

DMX CONSOLE

Fast Mode

Fast Model

Slow Mode

Slow Model

Soud Mode

Soud Mode

Sound Sense

10-100

Sound Sense

Pan Inverse

OFF

X Motor forward

ON

X Motor reversal

Tite Inverse

OFF

Y Motor forward

ON

Y Motor reversal

While balance

R 0-255

RED

G 0-255

Green

B 0-255

e

BLUE

W 0-255

WHITE

Default

confinrm

YES

Initialize

the factroy

Fixture

REST

YES

REST

16 channel

| channel | Features | Description |
| --- | --- | --- |
| 1 | Pan | 0-255 Adjust upward, turn forward, and reverse downward |
| 2 | Tilt | 0-255 Adjust upward, turn forward, and reverse downward |
| 3 | Total dimming | 0-255 Overall brightness adjustment |
| 4 | R dimming | 0-255 Red brightness adjustment |
| 5 | G dimming | 0-255 Green brightness adjustment |
| 6 | B dimming | 0-255 Blue brightness adjustment |
| 7 | W dimming | 0-255 White brightness adjustment |
| 8 | Strobe | 0-255 Strobe adjustment, from slow to fast |
| 9 | Motor running speed | 0-255 Pan&Tilt SPeed |
| 10 | 0-10 | Other channels work |
|  | 64-127 | Color combination change |
|  | 128-191 | Color jump |
|  | 192-255 | Color gradient |
| 11 | Color auto | Dyeing from slow to fast |
| 12 | Built-in effect | Built-in effect |
| 13 | Built-in effect speed | Built-in effect speed |
| 14 | Pan Fine | 0-255 Adjust upward, turn forward, and reverse downward |
| 15 | Tilt Fine | 0-255 Adjust upward, turn forward, and reverse downward |
| 16 | REST | 250-255 no affect; 250-255 REST |

9 channel

| channel | Features | Description |
| --- | --- | --- |
| 1 | Pan | 0-255 Adjust upward, turn forward, and reverse downward |
| 2 | Tilt | 0-255 Adjust upward, turn forward, and reverse downward |
| 3 | Total dimming | 0-255 Overall brightness adjustment |
| 4 | R dimming | 0-255 Red brightness adjustment |
| 5 | G dimming | 0-255 Green brightness adjustment |
| 6 | B dimming | 0-255 Blue brightness adjustment |
| 7 | W dimming | 0-255 White brightness adjustment |
| 8 | Strobe | 0-10 no strobe; 11-255 Strobe adjustment, from slow to fast |
| 9 | Motor running speed | 0-255 Pan&Tilt SPeed from slow to fast |

555222222