36x12w Led Moving Head Wash

USER MANUAL

Catalog

1.	Outside Size	3

2.	Main Function	3

3.	DMX profile	3

4.	Display Operation instruction	4

5.	Trouble Shooting	6

# Outside Size

# Main Function

- Input voltage AC 100V-240V/47-63HZ

- Power consumption：360W

- Lamp：RGBW 4IN1

- Control mode：DMX512/Auto/Sound/Master/Slave

- Channel mode：16CH

- Protection： IP20

- Size:    mm

- Net weight:     Kg

# DMX profile

- DMX profile

| 16CH | VALUE | FUNCTION |
| --- | --- | --- |
| CH1 | 0-255 | Pan |
| CH2 | 0-255 | Tilt |
| CH3 | 0-255 | Pan/Tilt speed(fast to slow) |
| CH4 | 0-255 | Dimmer |
| CH5 | 0-255 | R |
| CH6 | 0-255 | G |
| CH7 | 0-255 | B |
| CH8 | 0-255 | W |
| CH9 | 0-10 | No strobe |
|  | 11-255 | strobe slow to fast |
| CH10 | 0-255 | zoom |
| CH11 | 0-29 | No effect |
|  | 30-39 | R |
|  | 40-49 | G |
|  | 50-59 | B |
|  | 60-69 | Yellow |
|  | 70-79 | Pink |
|  | 80-89 | Cyan |
|  | 90-99 | RGBW |
|  | 100-109 | AUTO 1 (R,G,B,W,RG,RB,GB) |
|  | 110-119 | AUTO 2 (R↑,R↓,G↑,G↓,B↑,B↓,W↑,W↓,RG↑,RG↓,RB↑,RB↓,GB↑,GB↓) |
|  | 120-129 | AUTO 3(Rainbow effect: B,BG↑,BG,B↓G,G,GR↑,GR,G↓R,R,RB↑,RB,R↓B) |
|  | 130-255 | Reserved |
| CH12 | 0-255 | Auto effect speed |
| CH13 | 0-255 | Pan fine |
| CH14 | 0-255 | Tilt fine |
| CH15 | 0-250 | No effect |
|  | 251-255 | Reset(at least 5S) |
| CH16 | 0-255 | Reserved |

# Display Operation instruction

- MENU : top menu scroll or return to a previous menu option

- ENTER: enter to the next level menu or save current setting parameter

- UP: menu selection or parameter increments

- DOWN: menu selection or parameters decrease

Menu tree

| TOP | LEVEL 1 | LEVEL 2 |
| --- | --- | --- |
| DMX ADDRESS | 001-512 |  |
| SETTING | CHANNEL MODE | 16CH |
|  | PAN TILT FEEDBACK | ON/OFF |
|  | PAN INVERT | ON/OFF |
|  | TILT INVERT | ON/OFF |
|  | SWAP PAN TILT | ON/OFF |
|  | SOUND SENSITIVITY | 1-15-30 |
| DISPLAY | LANGUAGE | ENGLISH/中文 |
|  | DISPLAY INVERSE | ON/OFF |
|  | BACKLIGHT | PERMANENTLY ON/30S OFF |
| MANUALLY CONTROL | PAN | 0-255 |
|  | PAN FINE | 0-255 |
|  | TILT | 0-255 |
|  | TILT FINE | 0-255 |
|  | PAN&TILT SPEED | 0-255 |
|  | DIMMER | 0-255 |
|  | R | 0-255 |
|  | G | 0-255 |
|  | B | 0-255 |
|  | W | 0-255 |
|  | A | 0-255 |
|  | UV | 0-255 |
|  | STROBE | 0-255 |
|  | ZOOM | 0-255 |
|  | MACRO | 0-255 |
|  | MACRO SPEED | 0-255 |
| AUTO MODE | MASTER AUTO RUN | RUNNING…* |
|  | ALONE AUTO RUN | RUNNING…* |
|  | MASTER SOUND RUN | RUNNING…* |
|  | ALONE SOUND RUN | RUNNING…* |
| FACTORY DEFAULT | YES/NO |  |
| INFORMATION | SOFTWARE Vx.x |  |
| RESET | YES/NO |  |

Comment：

- When enter to the “*” position displayed on the menu, the fixture will automatically set as master and send data to external. Other fixtures can receive this data and will synchronously running without manually set as slave. When power cycle it will jump to the “*” position and running again. And in these position it will not return back to the default display after 30s.

- When return factory default only return to the char with under line parameter others will not change.

- 【SEND PARAMETERS】 can send setting parameters with under line via DMX cable and there will be an automatic reset of the other projectors after receiving parameter correctly.

# Trouble Shooting

| PROBLEM | REASON AND ACTION |
| --- | --- |
| The lighting can’t be started normally | - Check the power connection is correct or not. - The fuse is damage. - Please detect the voltage. - Power supply is damaged or incorrect connected. Call a qualified personnel to fix it. - Connection of control board is not correct. Call a qualified personnel to fix it. |
| Out of console’s control | - Please check the DMX connector and the power connection is connected correctly or not. When it having signal the screen panel green LED will blink.Please check the DMX address setting of lighting is correct or not. - Check【CHANNEL MODE】setting is correct or not. - Please check whether the DMX line is near to the high voltage wire or not. In that case, it will damage or interfere the DMX electric circuit. |
| Pan or tilt running incorrect | - Check feedback sensor adjust the position or cancel this function in 【PAN TILT FEEDBACK】 menu. |